Chip-Level Design of a Hardware Root-of-Trust for Firmware Authentication

Block Diagram of a Hardware ROT

Complete Module-Level and Pin-to-Pin Design Document

1. Introduction

This document describes the complete Hardware Root-of-Trust (RoT) architecture implemented for secure boot, operating strictly before CPU execution. The RoT ensures that only authenticated firmware (BIOS/bootloader) stored in flash memory is allowed to execute by holding the CPU in reset until verification succeeds.

This design is:

- Hardware-enforced (no software dependency)

- Deterministic and fail-safe

- Suitable for SoC secure boot reference implementations

2. Top-Level Module: rot_top

What it is

rot_top is the integration wrapper for the entire Root-of-Trust IP. It defines the security boundary of the RoT and exposes only minimal, safe control signals to the rest of the SoC.

Role

- Instantiates and connects all RoT submodules

- Routes firmware data and control signals

- Exposes CPU reset and boot status

- Prevents any direct external access to secure internals

Pin Description

Inputs

- clk : System clock used by RoT logic

- rst_n : Global reset (active low)

- boot_req : Indicates a boot request after power-on

- fw_data [N-1:0] : Firmware data stream from flash controller

- fw_valid : Indicates valid firmware data

- fw_last : Indicates last firmware word (optional)

Outputs

- cpu_reset_n : Controls CPU reset (active low)

- boot_done : Indicates boot decision completed

- boot_pass : Indicates firmware authentication success

- secure_mode : Indicates system is in secure boot mode

- debug_disable : Disables debug access during secure boot

3. Boot Controller FSM: boot_ctrl_fsm

What it is

A finite state machine that controls the entire secure boot flow. It is the central decision coordinator but does not perform cryptography or access secret keys.

Role

- Enforces strict boot sequencing

- Triggers authentication

- Waits for authentication result

- Commands allow / deny / lockdown actions

FSM States

- FSM_RESET

- FSM_IDLE

- FSM_AUTH_START

- FSM_AUTH_WAIT

- FSM_CHECK_RESULT

- FSM_BOOT_ALLOW

- FSM_BOOT_DENY

- FSM_LOCKDOWN

Pin Description

Inputs

- clk

- rst_n

- boot_req

- auth_done

- auth_pass

- lockdown_req

Outputs

- start_auth : Triggers authentication engine

- boot_done

- boot_pass

- cpu_allow : Allows CPU reset release

- enter_lockdown

4. Authentication Engine: auth_engine

What it is

A hardware authentication block that verifies firmware authenticity using a trusted reference from secure key storage.

Role

- Receives firmware data as a stream

- Computes hash or verifies signature (abstracted)

- Compares against trusted key/hash

- Reports pass/fail result

Pin Description

Inputs

- clk

- rst_n

- start_auth

- fw_data [N-1:0]

- fw_valid

- fw_last

- trusted_key [K-1:0]

Outputs

- auth_done : Authentication complete

- auth_pass : Authentication success

Notes

- Cryptographic strength is abstracted

- Focus is on functional correctness and control flow

5. Secure Key Storage: secure_key_storage

What it is

A protected, immutable storage block holding trusted root material such as public keys or golden hashes.

Role

- Acts as trust anchor

- Prevents software or external access

- Supplies keys only to auth engine

Pin Description

Inputs

- clk

- rst_n

Outputs

- trusted_key [K-1:0]

Properties

- No write interface

- OTP / ROM modeled

- Only readable internally

6. Firmware Data Interface: fw_data_if

What it is

Logical interface between external flash controller and RoT.

Role

- Supplies firmware data to authentication engine

- Decouples RoT from flash implementation

Signals

- fw_data

- fw_valid

- fw_last

7. Policy Engine: policy_engine

What it is

A rule-evaluation block that decides final boot action based on authentication result and system policy.

Role

- Interprets auth results

- Applies retry limits

- Requests lockdown if needed

Pin Description

Inputs

- auth_done

- auth_pass

- retry_count

Outputs

- policy_allow

- policy_deny

- lockdown_req

8. Retry Counter: retry_counter

What it is

A security counter that limits repeated authentication attempts.

Role

- Prevents brute-force attacks

- Triggers lockdown after threshold

Pin Description

Inputs

- clk

- rst_n

- auth_fail

Outputs

- retry_count

- retry_exceeded

9. Lockdown Controller: lockdown_ctrl

What it is

A fail-safe security module that enforces permanent denial on critical failures.

Role

- Overrides all boot logic

- Forces CPU reset

- Disables debug permanently

Pin Description

Inputs

- lockdown_req

Outputs

- lockdown_active

10. CPU Reset Control: cpu_reset_ctrl

What it is

Final enforcement point controlling CPU execution.

Role

- Holds CPU in reset until secure boot passes

- Reacts immediately to failures

Pin Description

Inputs

- cpu_allow

- lockdown_active

Outputs

- cpu_reset_n

11. Debug Control (Optional): debug_ctrl

What it is

Security block that controls debug access during secure boot.

Role

- Prevents debug-based attacks

- Locks debug permanently on failure

Pin Description

Inputs

- secure_mode

- lockdown_active

Outputs

- debug_disable

12. Summary

This RoT design:

- Operates strictly before CPU execution

- Enforces secure boot via hardware FSM

- Separates control, cryptography, and policy

- Is extensible and synthesizable

This document serves as a complete design reference for implementation, verification, and academic publication.