HARDWARE ROOT OF TRUST FOR SECURE SYSTEM OPERATION Complete Technical Project Documentation

RTL Design & Verification · Cadence EDA Tools · SHA-256 Cryptography · RTL-to-GDS Flow

| Document Attribute | Details |
| --- | --- |
| Project Title | Hardware Root of Trust (HRoT) for Secure System Operation |
| Document Type | Complete Technical Project Documentation |
| Domain | VLSI Design, Verification, Cryptography, Physical Design |
| HDL Used | SystemVerilog (IEEE 1800-2017) |
| Verification Methodology | Universal Verification Methodology (UVM 1.2) |
| Cryptographic Algorithm | SHA-256 (NIST FIPS 180-4) |
| Primary EDA Suite | Cadence Xcelium · Genus · Innovus |
| Design Flow | RTL → Synthesis → P&R → DRC/LVS → GDSII |

Document Structure — Sections at a Glance Chapter 1 · What is a Root of Trust — Definition, Need, and Importance Chapter 2 · Technologies Used — HDLs, Methodologies, and EDA Tools Chapter 3 · SHA-256 Cryptographic Algorithm — Theory and Hardware Implementation Chapter 4 · Module-by-Module Descriptions — All 8 RTL Modules in Detail Chapter 5 · RTL-to-GDS Flow — Complete VLSI Implementation Using Cadence Tools

# Chapter 1: Root of Trust (RoT) — Foundation of Hardware Security

## 1.1  What is a Root of Trust?

A Root of Trust (RoT) is a set of implicitly trusted functions in a computing system — a secure foundation from which all other security operations are derived. Unlike software-based security, which can be tampered with after the system starts, a hardware Root of Trust is embedded in silicon and is physically impossible to override by software.

Think of it this way: when you power on a device, the very first code that runs must be trusted. If that first code is compromised, every security check that follows is meaningless. The Root of Trust is that first, always-trusted piece of hardware logic.

Simple Definition A Root of Trust is a hardware component that is inherently trusted by the rest of the system. It provides verified, tamper-proof services — like firmware authentication — before any software runs. It answers one critical question at boot time: 'Is the firmware I am about to execute the authentic, unmodified original?'

## 1.2  Why is a Root of Trust Required?

Modern electronic devices — from smartphones and laptops to IoT sensors and automotive ECUs — run firmware that controls every hardware operation. Firmware is stored in flash memory, which is rewritable. This creates a serious security vulnerability:

| Attack Scenario | What the Attacker Does | Consequence Without RoT |
| --- | --- | --- |
| Firmware Replacement | Overwrites flash with malicious firmware | Malware executes before OS — total device compromise |
| Firmware Bit-Flip Attack | Uses physical fault injection to flip bits in flash | Modified firmware executes undetected |
| Supply Chain Attack | Tampers with firmware during manufacturing/shipping | Device ships with backdoor already installed |
| Rootkit Installation | Injects persistent malware into boot firmware | Malware survives OS reinstallation |
| Insecure OTA Update | Intercepts and replaces firmware over-the-air | Attacker controls device remotely |

Without a Root of Trust, the device has no way to verify whether the firmware it is about to execute is the authentic original or a malicious replacement. The device blindly runs whatever code is in flash memory.

The Core Problem RoT Solves ❌ Without RoT: Device powers on → Loads firmware from flash → Executes immediately (no verification) ✅ With RoT: Device powers on → RoT reads firmware → Computes SHA-256 hash → Compares with trusted hash → Hash match? → Allow execution ¦ Hash mismatch? → Block device permanently

## 1.3  Importance of Root of Trust

### ◆  1.3.1  Security Hierarchy — The Chain of Trust

A Root of Trust is the anchor of a Chain of Trust. Every component that boots after the RoT does so because the RoT vouches for it. If the RoT is trustworthy, every subsequent step in the boot sequence can be made trustworthy.

Power ON

│

▼

[RoT — Hardware, immutable, always trusted]

│   Verifies ↓

▼

[Boot Loader — authenticated by RoT]

│   Verifies ↓

▼

[Operating System — authenticated by Boot Loader]

│   Verifies ↓

▼

[Applications — authenticated by OS]

### ◆  1.3.2  Why Hardware RoT is Superior to Software RoT

| Aspect | Software RoT | Hardware RoT (This Project) |
| --- | --- | --- |
| Tamper Resistance | Can be modified by privileged software or OS | Implemented in silicon — cannot be modified |
| Boot Time Trust | Relies on software already running | Active before any software loads |
| Key Storage | Keys in RAM/flash — extractable | Keys in OTP/eFuse — physically protected |
| Attack Surface | Exposed to OS and application vulnerabilities | Isolated from software attack surface |
| Reliability | Can be bypassed by OS-level exploits | Bypass is physically impossible |

### ◆  1.3.3  Real-World Applications

- Smartphones — Android Verified Boot and Apple Secure Enclave both use hardware RoT.

- Laptops — UEFI Secure Boot with TPM (Trusted Platform Module) provides hardware RoT.

- Automotive ECUs — ISO 21434 mandates secure boot with hardware RoT for safety-critical systems.

- Cloud Servers — Google Titan and Microsoft Pluton chips implement data-centre hardware RoT.

- IoT Devices — ARM TrustZone and PSA Certified enforce RoT on resource-constrained hardware.

- Industrial Control Systems — Secure firmware authentication prevents industrial sabotage.

### ◆  1.3.4  Summary of Importance

| Importance Factor | Description |
| --- | --- |
| Foundation of All Security | Every security service in the system derives its trust from the RoT |
| Hardware Enforcement | Security decisions cannot be overridden by any software |
| Firmware Integrity | Guarantees that only authentic firmware executes on the device |
| Attack Prevention | Stops firmware replacement, brute-force, and persistent rootkit attacks |
| Regulatory Compliance | Required by NIST SP 800-193, IEC 62443, ISO 21434, PSA Certified |
| Device Lifecycle Security | Protects firmware from manufacture through end-of-life |

# Chapter 2: Technologies Used to Implement This Project

This project integrates four technology pillars: Hardware Description Languages, Verification Methodology, Cryptography, and Physical Design using Cadence EDA tools. Each is described below in full detail.

## 2.1  Hardware Description Languages (HDL)

### ◆  2.1.1  What is an HDL?

A Hardware Description Language is used to model and describe digital hardware at various levels of abstraction. Unlike C or Python which run sequentially, HDLs describe hardware that operates in parallel — because real circuits have thousands of gates all switching simultaneously.

| HDL | Level | Industry Usage |
| --- | --- | --- |
| Verilog | RTL + Gate | ASIC front-end design; widely used since 1985 |
| SystemVerilog | RTL + Verification | Industry-standard for both design and UVM verification |
| VHDL | RTL + Gate | European industry; safety-critical systems (DO-254) |
| SystemC | TLM / ESL | System-level modelling and architectural exploration |

### ◆  2.1.2  SystemVerilog — The Language Used in This Project

SystemVerilog (IEEE 1800-2017) is an extension of Verilog that adds powerful design and verification features. It is the industry-standard language used at companies like Intel, NVIDIA, Qualcomm, AMD, and Apple for chip design and verification.

| SystemVerilog Feature | How It Is Used in This Project |
| --- | --- |
| logic data type | Used for all RTL signals — replaces ambiguous Verilog wire/reg |
| always_ff | Used for all flip-flop inference — sequential logic blocks |
| always_comb | Used for all combinational logic — policy engine, comparator |
| typedef enum | Used for FSM state declarations — clean, readable state names |
| parameter / localparam | Used for key width (256), retry limit (3), data width (32) |
| module / port declaration | All 8 RTL modules declared with typed, directional ports |
| Packed arrays | Used for 256-bit key and hash vectors: logic [255:0] trusted_key |
| Non-blocking assignment (<=) | Used in all always_ff blocks to prevent race conditions |

Why SystemVerilog instead of plain Verilog? 1. always_ff / always_comb are synthesis-safe — the tool can immediately identify latches vs registers. 2. Enum types make FSM code readable and prevent illegal state assignments. 3. Packed structs and arrays allow clean description of 256-bit cryptographic data paths. 4. SystemVerilog supports the full UVM testbench — classes, interfaces, constraints, coverage.

## 2.2  Verification Methodology — UVM

### ◆  2.2.1  What is UVM?

UVM (Universal Verification Methodology) is the industry-standard framework for functionally verifying complex digital designs. It is based on SystemVerilog object-oriented programming and was standardised by Accellera. Every major semiconductor company uses UVM — it has replaced older, ad-hoc testbench approaches.

### ◆  2.2.2  Why UVM is Necessary

Manual, directed testing is insufficient for complex designs. A chip with millions of gates has billions of possible states — you cannot write a test for each one. UVM solves this through:

- Constrained Random Verification — automatically generates thousands of test stimulus combinations.

- Functional Coverage — tracks which design states and transitions have been exercised.

- Reusability — verification components (agents, drivers, monitors) are reused across projects.

- Scoreboards — automatically compare expected vs actual results for every transaction.

### ◆  2.2.3  UVM Testbench Architecture for This Project

| UVM Component | Class Base | Role in This Project |
| --- | --- | --- |
| uvm_test | uvm_test | Top-level test; selects which scenario to run (valid boot, fail, lockdown) |
| uvm_env | uvm_env | Container holding all sub-components of the verification environment |
| uvm_agent (active) | uvm_agent | Bundles driver + monitor + sequencer for the DUT interface |
| uvm_driver | uvm_driver | Drives fw_data, fw_valid, fw_last, boot_req signals to DUT |
| uvm_monitor | uvm_monitor | Passively observes auth_pass, boot_done, device_enable_n, lockdown_active |
| uvm_scoreboard | uvm_scoreboard | Compares DUT output against golden expected model |
| uvm_sequence | uvm_sequence | Defines firmware data streams — valid, tampered, multi-block |
| uvm_coverage | uvm_subscriber | Tracks FSM state coverage, toggle coverage, path coverage |

### ◆  2.2.4  Key Test Scenarios Implemented

- Valid firmware authentication — firmware matches trusted hash → device_enable_n = 0.

- Tampered firmware — one bit flipped → auth_pass = 0, device_enable_n = 1.

- Retry limit exhaustion — fail MAX_RETRY times → lockdown_active = 1.

- Lockdown persistence — verify lockdown survives multiple boot_req attempts.

- Recovery mode activation — retry exceeded, user requests recovery.

- Multi-block firmware — firmware > 512 bits requires multiple SHA-256 blocks.

- Reset during authentication — assert rst_n mid-operation → clean return to IDLE.

## 2.3  EDA Tools — Cadence Tool Suite

This project uses the Cadence digital design flow — a complete, integrated ecosystem of EDA tools covering every stage from RTL simulation through physical layout to tape-out.

### ◆  2.3.1  Cadence Xcelium — Simulation Engine

Xcelium is Cadence's industry-leading RTL simulator. It supports SystemVerilog, VHDL, and mixed-language designs. It features a parallel simulation engine that distributes workload across CPU cores for fast regression runs.

| Capability | Details |
| --- | --- |
| Languages Supported | SystemVerilog, Verilog, VHDL, SystemC, UVM |
| Simulation Mode | RTL, gate-level (with SDF), mixed-signal |
| UVM Support | Native UVM library integration — no external setup needed |
| Coverage Types | Code, functional, toggle, FSM, assertion coverage |
| Debug Features | Waveform dump (VCD/FSDB), interactive debug, signal tracing |
| Performance | Parallel simulation — up to 10x faster than single-thread VCS |

### ◆  2.3.2  Cadence Genus — Logic Synthesis

Genus converts the RTL (SystemVerilog) description into a gate-level netlist using standard cells from the foundry's cell library. It applies timing, area, and power optimisation simultaneously using machine-learning-guided algorithms.

| Synthesis Stage | What Genus Does |
| --- | --- |
| Elaboration | Reads RTL, resolves parameters, builds internal design hierarchy |
| Generic Synthesis | Boolean minimisation, constant propagation, technology-independent optimisation |
| Technology Mapping | Maps optimised logic to foundry standard cells (AND, OR, FF, MUX) |
| Timing Optimisation | Re-sizes cells, adds buffers to meet clock period constraints |
| Area Recovery | Removes redundant logic, reduces gate count after timing is met |
| Power Optimisation | Gate sizing, operand isolation, clock gating insertion |

### ◆  2.3.3  Cadence Innovus — Physical Design (Place and Route)

Innovus is the Cadence implementation platform for physical design. It handles everything from floorplanning through routing to GDSII export. Innovus supports multi-corner multi-mode (MCMM) analysis, enabling timing convergence across all PVT corners simultaneously.

| P&R Stage | Innovus Function |
| --- | --- |
| Floorplanning | Define die area, place macros, create power grid (VDD/VSS) |
| Placement | Place standard cells optimally — timing-driven, congestion-aware |
| Clock Tree Synthesis | Build balanced clock tree — minimise skew < 100 ps |
| Routing (NanoRoute) | Route all signal wires on metal layers — global then detailed |
| Post-Route Optimisation | Fix timing/DRC violations after routing |
| GDSII Export | Stream out final layout for foundry mask fabrication |

### ◆  2.3.4  Cadence Tempus — Static Timing Analysis (STA)

Tempus performs golden sign-off STA using post-layout extracted parasitics (SPEF). It analyses setup and hold timing across all process-voltage-temperature (PVT) corners to confirm the design meets timing under worst-case conditions.

### ◆  2.3.5  Cadence IMC (Incisive Metrics Center) / vManager — Coverage Analysis

IMC (now evolved into vManager) is Cadence's coverage analysis and management platform. After simulation regressions complete, IMC merges coverage databases from all test runs and provides a unified view of what has been tested and what remains untested.

| IMC / vManager Feature | Role in This Project |
| --- | --- |
| Coverage Database Merge | Combines coverage from all UVM test runs into one unified database |
| FSM Coverage | Verifies all 6 states (IDLE, START_AUTH, WAIT_AUTH, SUCCESS, FAIL, LOCKDOWN) are hit |
| Toggle Coverage | Checks every signal bit has been exercised both 0→1 and 1→0 |
| Functional Coverage | Tracks custom covergroups defined in UVM scoreboard/monitor |
| Coverage Exclusions | Marks unreachable code (dead states) as excluded from coverage targets |
| Regression Dashboard | Visual report showing coverage progression run-by-run |

## 2.4  Technology Stack — Complete Summary

| Layer | Technology / Tool | Version / Standard | Purpose |
| --- | --- | --- | --- |
| Design Language | SystemVerilog | IEEE 1800-2017 | RTL authoring for all 8 modules |
| Verification Language | SystemVerilog + UVM | UVM 1.2 | Testbench, sequences, coverage |
| Simulation | Cadence Xcelium | 23.x | RTL and gate-level simulation |
| Coverage Analysis | Cadence IMC / vManager | 23.x | Coverage merge and closure |
| Synthesis | Cadence Genus | 21.x | RTL → Gate netlist |
| Physical Design | Cadence Innovus | 21.x | Floorplan, Place, Route, CTS |
| Static Timing | Cadence Tempus | 21.x | STA sign-off, ECO |
| Physical Verification | Cadence Pegasus | 21.x | DRC, LVS, ERC |
| Cryptography | SHA-256 (custom RTL) | NIST FIPS 180-4 | Firmware hash computation |
| Constraints | SDC | SDC 2.1 | Timing constraints for synthesis + STA |
| Parasitics | SPEF | IEEE 1481 | Post-layout RC extraction |

# Chapter 3: SHA-256 Cryptographic Algorithm

SHA-256 is the cryptographic core of this project. It provides the mathematical guarantee that any modification to the firmware — even a single bit change — will produce a completely different hash output, making it detectable during authentication.

## 3.1  What is SHA-256?

SHA-256 (Secure Hash Algorithm — 256 bit) is a one-way cryptographic hash function standardised by the National Institute of Standards and Technology (NIST) in FIPS Publication 180-4 (2015). It belongs to the SHA-2 family.

| Property | Value / Description |
| --- | --- |
| Full Name | Secure Hash Algorithm — 256-bit output |
| Standard | NIST FIPS 180-4 (Federal Information Processing Standard) |
| Output Size | 256 bits (32 bytes) — always fixed, regardless of input size |
| Input Size | Unlimited — any message length from 0 to 2^64 - 1 bits |
| Block Size | 512 bits (64 bytes) per processing block |
| Word Size | 32 bits — all internal operations use 32-bit words |
| Number of Rounds | 64 compression rounds per 512-bit block |
| Security Level | 128-bit security — collision resistance 2^128 operations |

## 3.2  Why SHA-256 for Firmware Authentication?

| Property | Explanation | Security Value |
| --- | --- | --- |
| Deterministic | Same firmware always produces same hash | Reproducible verification result |
| Avalanche Effect | 1-bit change in firmware → ~50% of hash bits change | Any tampering is detected |
| Pre-image Resistant | Cannot reverse a hash to recover the firmware | Trusted hash cannot reveal firmware |
| Collision Resistant | Infeasible to create two firmwares with same hash | Attacker cannot forge a matching firmware |
| Fixed Output | Always 256 bits — simple hardware comparator | Constant-time comparison in RTL |
| Industry Trusted | Used in TLS, Bitcoin, UEFI Secure Boot, AWS HSM | Proven over decades in production |

## 3.3  SHA-256 Internal Constants

### ◆  3.3.1  Initial Hash Values (H0 – H7)

Eight 32-bit initial hash values are derived from the fractional parts of the square roots of the first 8 prime numbers (2, 3, 5, 7, 11, 13, 17, 19):

H0 = 0x6a09e667   // frac(sqrt(2))

H1 = 0xbb67ae85   // frac(sqrt(3))

H2 = 0x3c6ef372   // frac(sqrt(5))

H3 = 0xa54ff53a   // frac(sqrt(7))

H4 = 0x510e527f   // frac(sqrt(11))

H5 = 0x9b05688c   // frac(sqrt(13))

H6 = 0x1f83d9ab   // frac(sqrt(17))

H7 = 0x5be0cd19   // frac(sqrt(19))

### ◆  3.3.2  Round Constants K[0..63]

64 constant 32-bit words derived from the fractional parts of the cube roots of the first 64 primes:

K[0]  = 0x428a2f98    K[1]  = 0x71374491    K[2]  = 0xb5c0fbcf    K[3]  = 0xe9b5dba5

K[4]  = 0x3956c25b    K[5]  = 0x59f111f1    K[6]  = 0x923f82a4    K[7]  = 0xab1c5ed5

K[8]  = 0xd807aa98    K[9]  = 0x12835b01    K[10] = 0x243185be    K[11] = 0x550c7dc3

...   (64 constants total — K[0] through K[63])

K[60] = 0x90befffa    K[61] = 0xa4506ceb    K[62] = 0xbef9a3f7    K[63] = 0xc67178f2

## 3.4  SHA-256 Boolean Functions

SHA-256 uses six Boolean functions. ROTR^n means right-rotate by n bits. SHR^n means right-shift by n bits.

Ch(e, f, g)   =  (e AND f)  XOR  (NOT e AND g)

// Choose: where e=1 take f, where e=0 take g

Maj(a, b, c)  =  (a AND b)  XOR  (a AND c)  XOR  (b AND c)

// Majority: output the bit that appears in majority of a,b,c

SIGMA_0(a)    =  ROTR^2(a)  XOR  ROTR^13(a)  XOR  ROTR^22(a)

SIGMA_1(e)    =  ROTR^6(e)  XOR  ROTR^11(e)  XOR  ROTR^25(e)

sigma_0(w)    =  ROTR^7(w)  XOR  ROTR^18(w)  XOR  SHR^3(w)

sigma_1(w)    =  ROTR^17(w) XOR  ROTR^19(w)  XOR  SHR^10(w)

## 3.5  SHA-256 Algorithm — Step-by-Step

### ◆  Step 1 — Message Padding

Firmware (the message M) of length L bits is padded so its total length is congruent to 448 mod 512:

- Append a single '1' bit immediately after the last firmware bit.

- Append '0' bits until total length ≡ 448 (mod 512).

- Append the original message length L as a 64-bit big-endian integer.

Result: total padded message length is an exact multiple of 512 bits.

### ◆  Step 2 — Block Decomposition

The padded message is divided into N blocks of 512 bits: M(1), M(2), ..., M(N). Each 512-bit block is split into sixteen 32-bit words: W[0] through W[15].

### ◆  Step 3 — Message Schedule Expansion (Per Block)

Each block's 16 words are expanded to 64 words using:

for t = 16 to 63:

W[t] = sigma_1(W[t-2])  +  W[t-7]  +  sigma_0(W[t-15])  +  W[t-16]

(All additions are modulo 2^32)

### ◆  Step 4 — Initialise Working Variables

a = H0,  b = H1,  c = H2,  d = H3

e = H4,  f = H5,  g = H6,  h = H7

### ◆  Step 5 — 64-Round Compression Function

For each round t from 0 to 63:

T1 = h  +  SIGMA_1(e)  +  Ch(e,f,g)  +  K[t]  +  W[t]

T2 = SIGMA_0(a)  +  Maj(a,b,c)

h = g

g = f

f = e

e = d + T1          // mod 2^32

d = c

c = b

b = a

a = T1 + T2         // mod 2^32

### ◆  Step 6 — Update Hash State (After 64 Rounds)

H0 = H0 + a    H1 = H1 + b    H2 = H2 + c    H3 = H3 + d

H4 = H4 + e    H5 = H5 + f    H6 = H6 + g    H7 = H7 + h

(All additions modulo 2^32)

### ◆  Step 7 — Final Digest (After All N Blocks)

SHA256_DIGEST = H0 || H1 || H2 || H3 || H4 || H5 || H6 || H7

= 8 × 32-bit words = 256-bit final hash output

## 3.6  Hardware RTL Implementation of SHA-256

Implementing SHA-256 in RTL requires mapping each algorithmic step to synthesisable hardware constructs:

| Algorithm Component | RTL Implementation | Resource |
| --- | --- | --- |
| Message Buffer (W[0:15]) | 16 × 32-bit registers, load from fw_data bus | 512 flip-flops + MUX |
| Message Scheduler (W[16:63]) | Pipelined combinational logic + reg array | 64 × 32 registers + adder trees |
| Round Counter (t = 0..63) | 6-bit synchronous up-counter | 6 flip-flops + comparator |
| Working Variables (a–h) | Eight 32-bit registers, updated each clock cycle | 256 flip-flops |
| SIGMA_0, SIGMA_1 | Bit-rotation logic — pure combinational (wires) | Zero flip-flops |
| sigma_0, sigma_1 | Rotation + shift — pure combinational | Zero flip-flops |
| Ch() function | Two-level AND/XOR combinational gate network | ~96 gates |
| Maj() function | Three-level AND/XOR combinational gate network | ~96 gates |
| Modular adders | 32-bit carry-select adders — two per round (T1, T2) | ~64 full-adder stages |
| Hash Registers (H0–H7) | Eight 32-bit accumulator registers | 256 flip-flops |
| 256-bit Comparator | XOR tree reduction: (hash XOR trusted_key) == 0 | 256-gate XOR tree |

## 3.7  SHA-256 in the Authentication Flow

Flash Memory                                Secure Key Storage

│                                            │

│  fw_data[31:0], fw_valid, fw_last          │  trusted_key[255:0]

▼                                            ▼

┌─────────────────────────────────────────────────────────────┐

│                 Authentication Engine                       │

│                                                             │

│  fw_data → Message Buffer → Message Scheduler               │

│                                   │                         │

│                                   ▼                         │

│              64-Round Compression (a,b,c,d,e,f,g,h)         │

│                                   │                         │

│                                   ▼                         │

│                   Hash Registers H0–H7                      │

│                                   │                         │

│                    256-bit Digest ─┤◄── trusted_key         │

│                                   │                         │

│                          Comparator                         │

│                          │       │                          │

└──────────────────────────┼───────┼──────────────────────────┘

auth_pass  auth_done

# Chapter 4: Module Descriptions — Complete RTL Architecture

This chapter describes every RTL module present in the Hardware Root of Trust design. Each module is explained with its purpose, interface, internal working, practical example, and design rationale.

MODULE 01 · Central sequencer of the secure boot process Boot Controller FSM

### ◆  Purpose

The Boot Controller FSM is the master control unit that orchestrates the entire secure boot sequence. It decides when authentication starts, waits for the result, and transitions into appropriate security states. Without this module, no other module knows when to act.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | In | 1 | System clock — all state transitions on rising edge |
| rst_n | In | 1 | Active-low asynchronous reset — returns FSM to IDLE |
| boot_req | In | 1 | Boot request — asserted on power-on or system restart |
| auth_done | In | 1 | From Authentication Engine — verification complete |
| auth_pass | In | 1 | From Authentication Engine — 1=pass, 0=fail |
| lockdown_active | In | 1 | From Lockdown Controller — system in lockdown |
| start_auth | Out | 1 | To Authentication Engine — begin firmware verification |
| boot_done | Out | 1 | Signals system that boot decision has been made |

### ◆  FSM States and Transitions

| State | Entry Condition | Active Outputs | Next State |
| --- | --- | --- | --- |
| IDLE | Reset or previous boot complete | None | boot_req=1 → START_AUTH |
| START_AUTH | boot_req asserted | start_auth = 1 | Always → WAIT_AUTH |
| WAIT_AUTH | start_auth sent | None (waiting) | auth_done=1 → SUCCESS or FAIL |
| SUCCESS | auth_done & auth_pass=1 | boot_done = 1 | → IDLE |
| FAIL | auth_done & auth_pass=0 | boot_done = 1 | → IDLE (retry_counter increments) |
| LOCKDOWN | lockdown_active=1 | None (device stays off) | Stays in LOCKDOWN until reset |

### ◆  RTL Code Example

typedef enum logic [2:0] {

IDLE=3'b000, START_AUTH=3'b001, WAIT_AUTH=3'b010,

SUCCESS=3'b011, FAIL=3'b100, LOCKDOWN=3'b101

} state_t;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) state <= IDLE;

else        state <= next_state;

end

always_comb begin

case (state)

IDLE:       next_state = boot_req ? START_AUTH : IDLE;

START_AUTH: next_state = WAIT_AUTH;

WAIT_AUTH:  if (lockdown_active)  next_state = LOCKDOWN;

else if (auth_done)   next_state = auth_pass ? SUCCESS : FAIL;

else                  next_state = WAIT_AUTH;

SUCCESS:    next_state = IDLE;

FAIL:       next_state = IDLE;

LOCKDOWN:   next_state = LOCKDOWN;

endcase

end

### ◆  Practical Example — Timing Walk-Through

| Clock Cycle | Event | FSM State | Active Signal |
| --- | --- | --- | --- |
| Cycle 1 | System power-on, rst_n deasserted | IDLE | Waiting |
| Cycle 2 | boot_req = 1 asserted | IDLE → START_AUTH | start_auth = 1 |
| Cycle 3 | Authentication engine begins | START_AUTH → WAIT_AUTH | — |
| Cycle 3–N | SHA-256 computing firmware hash | WAIT_AUTH | — |
| Cycle N | auth_done = 1, auth_pass = 1 | WAIT_AUTH → SUCCESS | boot_done = 1 |
| Cycle N+1 | Boot complete, device enabled | SUCCESS → IDLE | — |

MODULE 02 · Tamper-resistant store for the trusted firmware hash Secure Key Storage

### ◆  Purpose

Secure Key Storage holds the 256-bit SHA-256 hash of the original, authentic firmware. This value is the trusted reference — the 'golden fingerprint' of legitimate firmware — programmed at manufacturing time and never changeable after that. In silicon this lives in OTP (One-Time Programmable) memory or eFuse arrays.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | In | 1 | System clock |
| rst_n | In | 1 | Active-low reset — zeroes root_key_reg |
| tb_load_key | In | 1 | SIMULATION ONLY — enables testbench to write key |
| tb_key_value | In | 256 | SIMULATION ONLY — testbench key value |
| trusted_key | Out | 256 | Trusted hash output → Authentication Engine |

### ◆  Internal Register

logic [255:0] root_key_reg;   // The secure key register

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n)

root_key_reg <= 256'b0;

else if (tb_load_key)              // Testbench simulation path only

root_key_reg <= tb_key_value;

// In real silicon: root_key_reg is hardwired from OTP/eFuse macros

end

assign trusted_key = root_key_reg;    // Continuous output

Important: Simulation vs. Silicon Behaviour In simulation: tb_load_key and tb_key_value allow the testbench to inject a key for testing. In silicon: These ports do not exist. root_key_reg is physically connected to OTP bit cells. Once the OTP is programmed at manufacturing, the key CANNOT be changed — ever. The trusted_key output is also not accessible via any debug or scan interface.

MODULE 03 · SHA-256 core — the security processor of the system Authentication Engine

### ◆  Purpose

The Authentication Engine is the most critical module. It receives firmware data word-by-word from flash memory, runs the complete SHA-256 algorithm on it, produces a 256-bit digest, and compares that digest against the trusted key from Secure Key Storage. This module determines whether firmware is authentic or not.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | In | 1 | System clock |
| rst_n | In | 1 | Active-low reset |
| start_auth | In | 1 | From Boot FSM — begin authentication |
| fw_data | In | 32 | Firmware word from flash — one 32-bit word per cycle when fw_valid=1 |
| fw_valid | In | 1 | High when fw_data contains valid firmware data |
| fw_last | In | 1 | High on the last valid word — triggers SHA-256 finalisation |
| trusted_key | In | 256 | Reference hash from Secure Key Storage |
| auth_done | Out | 1 | Authentication process has completed |
| auth_pass | Out | 1 | 1 = computed hash matches trusted key (firmware authentic) |

### ◆  Internal Architecture — Sub-Blocks

| Sub-Block | Function |
| --- | --- |
| Firmware Buffer (16×32) | Accumulates incoming 32-bit fw_data words into complete 512-bit SHA-256 blocks |
| Message Scheduler | Expands 16 message words (W[0:15]) to 64 scheduled words (W[0:63]) using sigma functions |
| Round Counter (6-bit) | Counts compression rounds 0 to 63; signals when a block is fully processed |
| Working Variables (a–h) | Eight 32-bit registers updated every round by Ch, Maj, SIGMA, T1, T2 logic |
| Hash Accumulators (H0–H7) | Eight 32-bit hash state registers; updated after each 512-bit block is processed |
| 256-bit Comparator | XOR-reduces (computed_hash XOR trusted_key) — outputs auth_pass when result is zero |

### ◆  Engine State Machine

| State | Description |
| --- | --- |
| IDLE | Waiting for start_auth; all registers hold reset values |
| COLLECT | Receiving fw_data words; buffering into 512-bit message block |
| PROCESS | Running 64-round SHA-256 compression on current block |
| CHECK_LAST | fw_last received — apply padding and process final block |
| COMPARE | Compare final 256-bit digest against trusted_key |
| DONE | Assert auth_done; hold auth_pass result until Boot FSM reads it |

### ◆  Practical Data Flow

Cycle 1:    start_auth = 1  →  engine exits IDLE

Cycle 2:    fw_valid=1, fw_data=0xABCD1234  →  W[0] = 0xABCD1234

Cycle 3:    fw_valid=1, fw_data=0x12345678  →  W[1] = 0x12345678

...

Cycle 17:   fw_valid=1, fw_data=0xXXXXXXXX, fw_last=1  →  W[15] loaded, last word

Engine enters PROCESS state

Message schedule expanded: W[16]..W[63] computed

64-round compression executes over 64 clock cycles

Hash state updated: H0..H7 finalised

Cycle 81:   Comparison: computed_hash == trusted_key ?

auth_pass = 1 (match) or 0 (mismatch)

auth_done = 1  →  Boot FSM advances

MODULE 04 · Brute-force attack prevention through attempt limiting Retry Counter

### ◆  Purpose

The Retry Counter tracks the number of consecutive authentication failures. Without it, an attacker could repeatedly try different firmware images indefinitely until one passes (or attempt to exploit corner cases in the verification logic). The counter limits this to MAX_RETRY attempts before triggering lockdown.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | In | 1 | System clock |
| rst_n | In | 1 | Active-low reset — clears counter to 0 |
| auth_fail | In | 1 | Derived signal: auth_done AND NOT auth_pass — one authentication failed |
| retry_count | Out | 2 | Current number of consecutive failures (0 to MAX_RETRY) |
| retry_exceeded | Out | 1 | Asserted when retry_count >= MAX_RETRY — triggers lockdown policy |

### ◆  RTL Implementation

parameter MAX_RETRY = 3;

localparam CNT_W = $clog2(MAX_RETRY + 1);  // = 2 bits for MAX_RETRY=3

logic [CNT_W-1:0] retry_count;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n)

retry_count <= '0;

else if (auth_fail && retry_count < MAX_RETRY)

retry_count <= retry_count + 1'b1;

// Counter saturates at MAX_RETRY — does not wrap

end

assign retry_exceeded = (retry_count >= MAX_RETRY);

### ◆  Practical Example — MAX_RETRY = 3

| Event | retry_count | retry_exceeded | System Action |
| --- | --- | --- | --- |
| Reset | 0 | 0 | Normal operation |
| Authentication Failure #1 | 1 | 0 | Boot denied, retry allowed |
| Authentication Failure #2 | 2 | 0 | Boot denied, retry allowed |
| Authentication Failure #3 | 3 | 1 | Lockdown triggered — device disabled |
| After Reset | 0 | 0 | Lockdown cleared — system restored |

MODULE 05 · Security decision-maker — translates results into actions Policy Engine

### ◆  Purpose

The Policy Engine is a purely combinational logic block that translates the authentication result and retry status into one of three distinct security decisions: allow boot, deny boot, or trigger lockdown. It acts as the judgment layer between the authentication result and the enforcement modules.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| auth_pass | In | 1 | 1 = authentication passed; 0 = failed |
| retry_exceeded | In | 1 | 1 = MAX_RETRY failures reached |
| policy_allow | Out | 1 | Allow device execution — sent to Device Controller |
| policy_deny | Out | 1 | Deny this boot attempt — retry may follow |
| policy_lockdown | Out | 1 | Trigger permanent lockdown — sent to Lockdown Controller |

### ◆  Decision Truth Table

| auth_pass | retry_exceeded | policy_allow | policy_deny | policy_lockdown | Action |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | Boot permitted — device enabled |
| 0 | 0 | 0 | 1 | 0 | Boot denied — retry available |
| 0 | 1 | 0 | 0 | 1 | Lockdown activated — permanent |
| 1 | 1 | 1 | 0 | 0 | Boot allowed (success before limit) |

### ◆  RTL Implementation (Combinational)

always_comb begin

policy_allow    = auth_pass;

policy_lockdown = ~auth_pass & retry_exceeded;

policy_deny     = ~auth_pass & ~retry_exceeded;

end

MODULE 06 · Permanent security enforcement after repeated failures Lockdown Controller

### ◆  Purpose

The Lockdown Controller converts the momentary policy_lockdown pulse from the Policy Engine into a permanent, latched lockdown_active signal. Once activated, the device cannot boot under any circumstances — only a physical hardware reset (rst_n) can clear the lockdown state. This prevents attackers from cycling the device to try more firmware attempts.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | In | 1 | System clock |
| rst_n | In | 1 | Active-low reset — only way to clear lockdown_active |
| policy_lockdown | In | 1 | From Policy Engine — assert to enter lockdown |
| lockdown_active | Out | 1 | Permanent lockdown flag — sent to Boot FSM and Device Controller |

### ◆  RTL Implementation — Set-Only Register

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n)

lockdown_active <= 1'b0;      // Reset clears lockdown

else if (policy_lockdown)

lockdown_active <= 1'b1;      // Once set, cannot be cleared by software

// No else clause — value is preserved once set (set-only register)

end

Design Intent — Why Set-Only? The lockdown register has NO software-writable clear path. Only rst_n (hardware reset pin) can clear it — requiring physical access to the device. This means an attacker who has triggered 3 failures cannot clear the lockdown via any software exploit, debug interface, or power cycle — they must have physical access and perform a hard reset.

MODULE 07 · Safe firmware repair path after authentication failure Recovery Mode Controller

### ◆  Purpose

Recovery Mode provides a controlled path for firmware repair when the retry limit has been exceeded. Without it, a device with a corrupted (but not malicious) firmware image would become permanently unusable. Recovery Mode allows legitimate firmware to be reloaded while maintaining security — the new firmware must still pass SHA-256 authentication.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| retry_exceeded | In | 1 | Retry limit reached — recovery becomes eligible |
| recovery_request | In | 1 | External signal from user button or trusted recovery interface |
| recovery_active | Out | 1 | Recovery mode active — enables firmware update interface |

### ◆  Recovery Flow

Normal boot fails 3 times  →  retry_exceeded = 1  →  lockdown triggered

User presses recovery button  →  recovery_request = 1

recovery_active = 1  →  firmware update interface (USB/UART/SPI) enabled

Device execution REMAINS disabled during recovery

User uploads new firmware image

Authentication Engine runs SHA-256 on new firmware

Hash matches?  →  device_enable_n = 0  →  device boots normally

Hash fails?    →  recovery rejects firmware — device stays disabled

Real-World Analogy Recovery Mode is like a phone's fastboot / recovery mode. The device is locked (no normal boot) but a trusted repair interface is available. New firmware must be cryptographically verified even in recovery — preventing attackers from using recovery as a bypass path to install malicious firmware.

MODULE 08 · Final hardware gate — allows or permanently blocks device execution Device Controller

### ◆  Purpose

The Device Controller is the last and most physically significant module. It controls the device_enable_n signal that is connected directly to the processor's reset or enable pin. The device can only start execution when device_enable_n is asserted low — and this only happens when authentication has passed and no lockdown is active.

### ◆  Interface Signals

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| allow | In | 1 | From Policy Engine — policy_allow; authentication passed |
| lockdown_active | In | 1 | From Lockdown Controller — permanent lockdown flag |
| device_enable_n | Out | 1 | To processor — active LOW; 0=enabled, 1=disabled |

### ◆  RTL Logic — Lockdown Takes Highest Priority

always_comb begin

if (lockdown_active)         // Highest priority — permanent disable

device_enable_n = 1'b1;

else if (allow)              // Authentication passed

device_enable_n = 1'b0;  // Enable device

else                         // Default — failed or waiting

device_enable_n = 1'b1;  // Keep device disabled

end

### ◆  All Operating Conditions

| Condition | allow | lockdown_active | device_enable_n | Result |
| --- | --- | --- | --- | --- |
| Firmware authenticated | 1 | 0 | 0 | Device starts execution |
| Authentication failed | 0 | 0 | 1 | Device stays off — retry possible |
| Lockdown active | 0 | 1 | 1 | Device permanently off |
| Lockdown + pass (impossible in HW) | 1 | 1 | 1 | Lockdown wins — device stays off |
| Reset state | 0 | 0 | 1 | Device off — waiting for boot |

The device_enable_n output connects directly to the processor's RESETN or ENABLE pin, making this a true hardware enforcement — no software path can assert this signal without passing through the authentication chain.