Technologies Used in the Project

Hardware Root of Trust for Secure System Operation

This project integrates multiple technologies from hardware design, verification, cryptography, and semiconductor physical design. The system is implemented using RTL design and verification techniques commonly used in VLSI industry workflows.

1. Hardware Description Languages (HDL)

What is an HDL

A Hardware Description Language is used to describe digital hardware circuits at various levels of abstraction.

Unlike software programming languages, HDL describes parallel hardware behavior.

Common HDLs used in industry include:

| HDL | Usage |
| --- | --- |
| Verilog | Digital design |
| SystemVerilog | Advanced design and verification |
| VHDL | Alternative hardware description |

Your project uses SystemVerilog.

SystemVerilog

SystemVerilog is an extension of Verilog that supports both:

- RTL design

- advanced verification

It is widely used in semiconductor companies such as:

- Intel

- NVIDIA

- Qualcomm

- AMD

Why SystemVerilog was used

SystemVerilog provides features useful for large designs.

Strong data types

Example:

logic

bit

int

These prevent common design errors.

Advanced constructs

SystemVerilog supports:

always_ff

always_comb

always_latch

These clearly specify hardware behavior.

Enumerated states

Used in FSM design:

typedef enum logic

This improves readability and reliability.

Parameterized design

Allows scalable modules.

Example:

parameter DATA_W = 32

parameter KEY_W = 256

RTL Design

What is RTL

RTL stands for Register Transfer Level.

RTL describes how data moves between registers and how logic processes that data.

Example:

register → combinational logic → register

Your entire Root of Trust architecture is implemented at RTL.

RTL Modules in the Project

The design contains the following RTL modules:

- Boot Controller FSM

- Secure Key Storage

- Authentication Engine

- Retry Counter

- Policy Engine

- Lockdown Controller

- Recovery Mode Controller

- Device Controller

- Top-level integration module

Each module is designed as a synthesizable RTL block.

2. UVM Verification Methodology

What is UVM

UVM stands for Universal Verification Methodology.

It is a standardized framework for verifying complex digital designs.

UVM is based on SystemVerilog object-oriented programming features.

Why UVM is used

Large semiconductor chips contain millions of gates.

Manual testing is not sufficient.

UVM provides:

- reusable verification components

- constrained random testing

- automated coverage tracking

UVM Architecture

A typical UVM testbench contains the following components:

Test

↓

Environment

↓

Agent

↓

Driver

↓

DUT

Main UVM Components

Test

Controls the simulation scenario.

Environment

Top-level container for verification components.

Agent

Handles communication with the DUT.

Driver

Drives stimulus signals into the design.

Example signals in your project:

fw_data

fw_valid

fw_last

boot_req

Monitor

Observes DUT outputs.

Example:

auth_pass

boot_done

device_enable_n

Scoreboard

Compares expected results with actual results.

Coverage

Measures how much of the design has been tested.

3. Cryptographic Technology

Cryptography in the Project

Your system uses cryptographic hashing to verify firmware integrity.

The algorithm used is:

SHA-256

SHA-256 Algorithm

SHA-256 stands for:

Secure Hash Algorithm – 256 bit

It is a cryptographic hash function defined by NIST.

It converts input data into a 256-bit hash value.

Why SHA-256 is used

SHA-256 provides:

- strong collision resistance

- secure fingerprinting

- widely trusted cryptographic standard

It is used in:

- secure boot systems

- blockchain

- TLS encryption

- digital signatures

SHA-256 Processing Steps

1. Message Padding

Input data is padded to make its length a multiple of 512 bits.

2. Block Processing

The message is divided into 512-bit blocks.

3. Message Expansion

Each block is expanded into 64 words.

4. Compression Function

The algorithm performs 64 rounds of operations.

Operations include:

- bitwise XOR

- bit rotation

- modular addition

5. Final Hash

After processing all blocks, the result is a 256-bit hash.

How SHA-256 is used in your project

Firmware

↓

SHA-256 Engine

↓

Computed Hash

↓

Compare with Trusted Hash

If both hashes match:

Firmware is authentic

4. RTL to GDS Flow

Your project follows a standard ASIC design flow.

RTL-to-GDS flow converts high-level RTL code into a manufacturable chip layout.

Step 1: RTL Design

Tools:

Cadence Xcelium

Synopsys VCS

Design is written in SystemVerilog.

Step 2: Functional Verification

Verification ensures the RTL behaves correctly.

Tools:

Cadence Xcelium

Mentor Questa

UVM testbench verifies:

- firmware authentication

- retry logic

- lockdown behavior

- device enable logic

Step 3: Logic Synthesis

RTL code is converted into a gate-level netlist.

Tool:

Cadence Genus

Output:

Gate-level netlist

Using standard cell libraries.

Step 4: Floorplanning

Chip layout planning begins.

Tool:

Cadence Innovus

Tasks include:

- defining chip size

- placing macros

- allocating power grids

Step 5: Placement

Standard cells are placed in the layout.

Goal:

- optimize area

- minimize routing congestion

Step 6: Clock Tree Synthesis (CTS)

Clock distribution network is built.

Goals:

- minimize clock skew

- ensure timing consistency

Step 7: Routing

Connections between cells are created.

Types of routing:

- global routing

- detailed routing

Step 8: Timing Analysis

Static Timing Analysis ensures the design meets timing constraints.

Tool:

Cadence Tempus

Checks:

- setup time

- hold time

- clock timing

Step 9: Physical Verification

Ensures layout correctness.

Checks include:

DRC – Design Rule Check

LVS – Layout vs Schematic

Tools:

Cadence Pegasus

Mentor Calibre

Step 10: GDSII Generation

Final chip layout is generated.

Output file:

GDSII

This file is sent to semiconductor fabrication.

5. Industry Tools Used

Common Cadence tools in ASIC design flow:

| Stage | Tool |
| --- | --- |
| RTL Simulation | Xcelium |
| Verification | Xcelium / UVM |
| Synthesis | Genus |
| Physical Design | Innovus |
| Timing Analysis | Tempus |
| Physical Verification | Pegasus |

6. Integration of Technologies

Your project integrates multiple engineering domains:

Cryptography

+

Hardware Design

+

Verification

+

Physical Design

Result:

Secure hardware Root of Trust architecture

Final Summary

This project combines SystemVerilog RTL design, UVM verification methodology, SHA-256 cryptographic authentication, and industry-standard ASIC design flow to implement a secure hardware Root of Trust that verifies firmware integrity before allowing device execution.

If you want, I can also give you a clean “Technology Stack” section for your website that looks like a professional semiconductor product page.